# Winter School Project

Team Members:

- Ritisha Bajaj

- Sahla Muneer

- Lahan Nazar

- Nihar K

- Indraneel Menon

- Bhargav Amudula

- Sai Ashrith

- Kanishka Singh

- Suhani Gupta

- Amrit Jeriath

1. Module Overview: Vision-Based Object Detection and Inverse Kinematics

This module handles the perception and kinematic computation part of the robotic arm system. The objective is to:

- Detect a black spherical object using a live camera feed

- Estimate the object’s position in the camera frame

- Convert the detected position into Cartesian coordinates

- Compute the required joint angles of a 3-DOF robotic arm using inverse kinematics

The computed joint angles are then used by the control module to replicate the object’s motion using the robotic arm.

2. Objective of This Module

The goals of this module are:

- Real-time detection of a black ball from a webcam feed

- Robust estimation of the object centroid

- Mapping image-space coordinates to arm workspace coordinates

- Computing three joint angles (base, shoulder, elbow) using inverse kinematics

This module operates continuously and updates joint angles frame-by-frame to enable smooth motion replication.

3. Software Tools and Libraries Used

- Python 3

- OpenCV (cv2) for image processing and camera handling

- NumPy for numerical operations

- Math library for trigonometric and geometric computations

4. Camera Initialization and Frame Acquisition

A live video stream is captured using the default webcam:

cam = cv.VideoCapture(0)

The camera resolution is obtained dynamically:

fw = int(cam.get(cv.CAP_PROP_FRAME_WIDTH))

fh = int(cam.get(cv.CAP_PROP_FRAME_HEIGHT))

Each frame is:

- Resized to the camera’s native resolution

- Smoothed using Gaussian blur to reduce noise

This preprocessing step improves thresholding and contour detection reliability.

5. Object Detection Methodology

5.1 Grayscale Conversion and Thresholding

Each frame is converted to grayscale and thresholded using binary inversion:

gray = cv.cvtColor(frame, cv.COLOR_BGR2GRAY)

ret, img = cv.threshold(gray, 80, 255, cv.THRESH_BINARY_INV)

This isolates the black ball as a white region against a dark background.

5.2 Contour Detection

Contours are extracted from the thresholded image:

contours, _ = cv.findContours(img, cv.RETR_EXTERNAL, cv.CHAIN_APPROX_SIMPLE)

- Only external contours are considered

- The contour with the maximum area is assumed to be the target object

A bounding box is drawn around the detected object for visualization.

6. Centroid Computation

To compute the object’s centroid, the detected contour is drawn on a blank canvas. The centroid is calculated as the mean of all active pixels belonging to the object.

Function Used:

def find_centroid(canvas, num_active_pixel):

Steps:

- Traverse the canvas pixel-by-pixel

- Store coordinates of all white pixels

- Compute centroid using mean position

This approach provides a stable centroid estimate even if the object shape slightly deforms.

7. Coordinate Mapping

The centroid position is converted from image coordinates to scaled Cartesian coordinates suitable for inverse kinematics:

x = centroid[0] * 2 / 3

y = centroid[1] * 2 / 3

z = num_active_pixel / 500

- x and y are derived from image-space centroid

- z is estimated using the number of active pixels, acting as a depth approximation

This allows approximate 3D positioning using a single camera.

8. Inverse Kinematics Formulation

The robotic arm is modeled as a 3-DOF manipulator with:

- One base rotation joint

- Two planar rotational joints

Link lengths:

- L1 = 9 units

- L2 = 7.5 units

8.1 Base Angle (θ₁)

The base rotation aligns the arm with the object in the horizontal plane:

θ1= tan-1(zx)

8.2 Planar Reduction

After base rotation, the problem reduces to a 2D inverse kinematics problem:

r=x2+z2

s=y

d=r2+s2

8.3 Elbow Angle (θ₃)

Using the cosine law:

D=d2-L12-L222L1L2

θ3=tan-1(1-D2D)

An elbow-down configuration is chosen for mechanical stability.

8.4 Shoulder Angle (θ₂)

ϕ=tan-1sr

ψ=tan-1(L2sinθ3L1+ L2cos θ3)

8.5 Angle Conversion

All angles are converted from radians to degrees before output.

9. Real-Time Visualization

For debugging and validation, the following are displayed live:

- Detected centroid

- Bounding box around the object

- Computed joint angles overlaid on the video frame

This allows immediate verification of detection accuracy and kinematic correctness.

10. Assumptions Made

- The object is a black spherical ball

- Background is relatively uniform

- Lighting conditions are stable

- Depth (z) is approximated using object area

- The robotic arm operates within its reachable workspace

11. Limitations

- Depth estimation is approximate due to single-camera setup

- Sudden lighting changes may affect thresholding

- Occlusion of the object can lead to centroid errors

- Workspace constraints may cause inverse kinematics failure

12. Outcome of This Module

- Successful real-time detection of the target object

- Smooth centroid tracking across frames

- Accurate computation of joint angles for arm replication

- Continuous angle updates suitable for closed-loop control